🌱 Project Idea: Smart Soil Moisture Watering System

What it does:

- Reads soil moisture from a sensor

- Sends telemetry to the cloud

- Uses serverless logic to decide when to water

- Turns a water pump (or LED for demo) on/off automatically

🧰 Components

- Microcontroller: ESP32 Dev Board (or Arduino + WiFi module)

- Sensor: Capacitive Soil Moisture Sensor v1.2

- Actuator: 5V Relay Module (controls pump or LED)

- Pump (optional): Mini Submersible Water Pump 3–6V

- Jumper wires, breadboard

🧠 Architecture

- Device reads soil moisture

- Sends data to Azure IoT Hub

- Azure Functions processes data

- Function sends command back (ON/OFF)

- Device turns relay (pump) on/off

🔌 Device Code (ESP32 – Arduino style)

#include <WiFi.h>
#include <PubSubClient.h>

const char* ssid = "YOUR_WIFI";
const char* password = "YOUR_PASSWORD";

const char* mqtt_server = "YOUR_IOTHUB.azure-devices.net";
const int soilPin = 34;
const int relayPin = 5;

WiFiClient espClient;
PubSubClient client(espClient);

void setup() {
  pinMode(relayPin, OUTPUT);
  digitalWrite(relayPin, LOW);

  Serial.begin(115200);
  WiFi.begin(ssid, password);

  while (WiFi.status() != WL_CONNECTED) {
    delay(500);
  }

  client.setServer(mqtt_server, 1883);
}

void loop() {
  int moisture = analogRead(soilPin);

  char payload[50];
  sprintf(payload, "{\"moisture\": %d}", moisture);

  client.publish("devices/device1/messages/events/", payload);

  delay(10000); // send every 10s
}

☁️ Azure Setup

1. Create IoT Hub

- Go to Azure Portal

- Create → IoT Hub

- Add a device (get connection string)

2. Create Azure Function

Trigger: IoT Hub Event

Example (JavaScript):

module.exports = async function (context, eventHubMessages) {
    for (const message of eventHubMessages) {
        const moisture = message.moisture;

        if (moisture < 300) {
            context.log("Soil dry → Turn ON pump");
            // send command to device
        } else {
            context.log("Soil wet → Turn OFF pump");
        }
    }
};

3. Send Command Back to Device

Use Direct Methods or Cloud-to-Device messages:

Example idea:

- If dry → send "ON"

- If wet → send "OFF"

🔁 Device: Listen for Commands

Add this to your ESP32:

void callback(char* topic, byte* payload, unsigned int length) {
  String message;

  for (int i = 0; i < length; i++) {
    message += (char)payload[i];
  }

  if (message == "ON") {
    digitalWrite(relayPin, HIGH);
  } else if (message == "OFF") {
    digitalWrite(relayPin, LOW);
  }
}