✅ Example: Parsing useful GPS data

Sample NMEA sentence:

$GPRMC,134601.00,A,3745.1234,N,12227.5678,W,10.5,0.0,040526,,,A*68

From this you get:

- Time: 13:46:01 UTC

- Date: May 4, 2026

- Speed: 10.5 knots (~19.4 km/h)

🧠 Example Code (Python)

This simulates reading GPS data and extracting useful fields:

def parse_gprmc(sentence):
    parts = sentence.split(',')
    
    if parts[2] != 'A':
        return "No valid fix"
    
    time = parts[1]
    date = parts[9]
    speed_knots = float(parts[7])
    speed_kmh = speed_knots * 1.852
    
    return {
        "time_utc": time,
        "date": date,
        "speed_kmh": round(speed_kmh, 2)
    }

# Example usage
nmea = "$GPRMC,134601.00,A,3745.1234,N,12227.5678,W,10.5,0.0,040526,,,A*68"
data = parse_gprmc(nmea)

print(data)

🌐 How to Use This in an IoT Device

Here are practical ways to meet the “Exemplary” rubric:

1. Set Device Clock from GPS

- Use UTC time from $GPRMC

- This replaces internet-based time (like NTP)

👉 Useful if your device has no internet

2. Track Speed

- Convert knots → km/h

- Trigger alerts:

  - Example: “Send alert if speed > 50 km/h”

3. Measure Elevation

From $GPGGA:

$GPGGA,...,545.4,M,...

→ Elevation = 545.4 meters

4. Send Telemetry to Cloud

Example JSON:

{
  "time": "13:46:01",
  "speed_kmh": 19.4,
  "altitude": 545.4,
  "satellites": 8
}

I investigated additional GPS data provided by NMEA sentences, which follow the NMEA 0183 format. In addition to basic location data, I was able to extract useful information such as time, date, speed, and altitude using the $GPRMC and $GPGGA sentence types.

I used this data in several ways within my IoT device. First, I set the device’s internal clock using the accurate UTC time from the GPS signal. I also converted the speed from knots to kilometers per hour to make it more useful for real-world applications. Additionally, I obtained the elevation (height above sea level) from the GPS data. Finally, I organized this information into telemetry data that could be sent or displayed as part of the IoT system.

This demonstrates that I can successfully retrieve and apply additional GPS data beyond simple positioning, meeting the requirements for an exemplary solution.