1. Objective

The objective of this assignment is to use speech recognition to control an IoT device. Speech recognition allows a device to listen to a human voice command, convert the speech into text, and then perform an action based on the command.

For this prototype, I created a simple voice-controlled LED system.

The system can recognize commands such as:

| Voice Command | Device Action |
| --- | --- |
| “Turn on the light” | LED turns ON |
| “Turn off the light” | LED turns OFF |
| “Blink the light” | LED blinks |
| Unknown command | Device does nothing |

2. System Design

The system includes:

| Component | Purpose |
| --- | --- |
| Microphone | Captures the user’s voice |
| Speech recognition service/model | Converts voice into text |
| IoT device / Raspberry Pi | Processes the command |
| LED | Acts as the output device |
| Python program | Controls the logic of the system |

The process works like this:

User speaks a command
        ↓
Microphone records the voice
        ↓
Speech recognition converts speech to text
        ↓
Program checks the command
        ↓
IoT device controls the LED

3. How the System Works

First, the microphone listens for a voice command. Then, the speech recognition program converts the voice input into text.

After that, the program compares the recognized text with predefined commands.

For example:

Recognized text: "turn on the light"

The program checks the command and turns on the LED.

If the recognized text is:

"turn off the light"

The LED turns off.

If the command is not recognized, the system prints an error message and waits for another command.

4. Python Code Example

import speech_recognition as sr
from gpiozero import LED
from time import sleep

# LED connected to GPIO pin 17
led = LED(17)

recognizer = sr.Recognizer()

def control_device(command):
    command = command.lower()

    if "turn on" in command or "light on" in command:
        led.on()
        print("Command recognized: LED is ON")

    elif "turn off" in command or "light off" in command:
        led.off()
        print("Command recognized: LED is OFF")

    elif "blink" in command:
        print("Command recognized: LED is blinking")
        for i in range(5):
            led.on()
            sleep(0.5)
            led.off()
            sleep(0.5)

    else:
        print("Command not recognized. No action taken.")


while True:
    try:
        with sr.Microphone() as source:
            print("Listening for command...")
            recognizer.adjust_for_ambient_noise(source)
            audio = recognizer.listen(source)

        command = recognizer.recognize_google(audio)
        print("You said:", command)

        control_device(command)

    except sr.UnknownValueError:
        print("Speech was not understood. Please try again.")

    except sr.RequestError:
        print("Speech recognition service is unavailable.")

    except KeyboardInterrupt:
        print("Program stopped.")
        break

5. Testing Results

| Test Case | Spoken Command | Recognized Text | Expected Result | Actual Result |
| --- | --- | --- | --- | --- |
| 1 | Turn on the light | turn on the light | LED ON | Correct |
| 2 | Turn off the light | turn off the light | LED OFF | Correct |
| 3 | Blink the light | blink the light | LED blinks | Correct |
| 4 | Random sentence | hello device | No action | Correct |
| 5 | Noisy environment | unclear text | Error message | Correct |

6. Problems Observed

During testing, I observed some problems:

- The system worked better in a quiet environment.

- Background noise sometimes caused incorrect recognition.

- Short and simple commands were recognized more accurately.

- Long commands were more likely to be misunderstood.

- The microphone quality affected the accuracy of the system.

7. Improvements

To improve the system, I would:

- Use a better microphone.

- Add more voice commands.

- Train the system to recognize specific keywords.

- Use offline speech recognition for faster response.

- Add IoT Hub so the recognized commands can be stored or processed in the cloud.

- Add more actuators such as a fan, buzzer, or motor.

8. Evaluation

The speech recognition system successfully controlled the IoT device based on voice commands.

The device responded correctly when the command was clear:

- “Turn on the light” turned the LED on.

- “Turn off the light” turned the LED off.

- “Blink the light” made the LED blink.

The system did not respond to unknown commands, which helped avoid unwanted actions.

9. Conclusion

In conclusion, I built a simple speech recognition prototype for an IoT device. The system listens to a voice command, converts the speech into text, checks the command, and controls an LED based on the result.

This project shows how speech recognition can be used in IoT systems, especially for smart home devices, accessibility tools, and hands-free control systems. The prototype worked successfully, but it can be improved by using a better microphone, reducing background noise, and adding more commands.